The reference pixel array refMain[ x ], with x=-nS..2*nS is specified as follows.

–	If intraPredMode is equal or greater than 18,

refMain[ x ] = p[ −1+x, −1 ], with x=0..nS		(8-1)

- If intraPredAngle is less than 0,

refMain[ x ] = p[ −1, −1+( ( x*invAngle+128 )>>8 ) ], with x=( nS*intraPredAngle ) >>5..−1	(8-2)

- Otherwise,

refMain[ x ] = p[ −1+x, −1 ], with x=nS+1..2*nS		(8-3)

Otherwise,

refMain[ x ] = p[ −1, −1+x ], with x=0..nS		(8-4)

- If intraPredAngle is less than 0,

refMain[ x ] = p[ −1+( ( x*invAngle+128 )>>8 ), −1 ], with x=( nS*intraPredAngle ) >>5..−1	(8-5)

- Otherwise,

refMain[ x ] = p[ −1, −1+x ], with x=nS+1..2*nS		(8-6)

The values of the prediction samples predSamples[ x, y ], with x, y = 0..nS−1 are derived by the following procedures.

–	The index variable iIdx and the multiplication factor iFact are derived by

iIdx = ( ( y + 1 )*intraPredAngle ) >> 5		(8-7)

iFact = ( ( y + 1 )*intraPredAngle ) && 31		(8-8)

–	Depending on the value of iFact, the following applies.

- If iFact is not equal to 0, the value of the prediction samples predSamples[ x, y ] is derived by

predSamples[ x, y ] = ( ( 32 – iFact )*refMain[ x+iIdx+1 ] + iFact*refMain[ x+iIdx+2] + 16 ) >> 5	(8-9)

- Otherwise, the value of the prediction samples predSamples[ x, y ] is derived by

predSamples[ x, y ] = refMain[ x+iIdx+1 ]		(8-10)

If intraPredMode is less than 18, the value of prediction samples predSamples[ x, y ] is swapped by that of predSamples[ y, x ] for y=0..nS-2, x=y+1..nS-1.