Modification – Version 1

#### Transformation process

Inputs of this process are:

–	a variable nS specifying the sample size of scaled transform coefficients,

–	an array of scaled transform coefficients x with elements xi, with i=0..nS-1.

–	a transform type variable trType

Output of this process is an array of samples of the residual samples y with elements yi, with i = 0..nS-1.

Depending on trType, the following applies:

–	If nS is equal to 4 and trType is equal to 1, the following ordered steps apply:

- A set of intermediate values c0, c1, c2 and c3 is calculated as follows:

c0 = x0 + x2		(8-1)
c1 = x2 + x3		(8-2)
c2 = x0 – x3		(8-3)
c3 = 74 * x1		(8-4)

- The output values yi with i = 0..3 are then specified as follows:

y0 = 29*c0 + 55*c1 + c3		(8-5)
y1 = 55*c2 – 29*c1 + c3		(8-6)
y2 = 74*( x0 - x2 + x3 ) 		(8-7)
y3 = 55*c0 + 29 * c2 – c3 		(8-8)

Modification – Version 2

#### Transformation process

Inputs of this process are:

–	a variable nS specifying the sample size of scaled transform coefficients,

–	an array of scaled transform coefficients x with elements xi, with i=0..nS-1.

–	a transform type variable trType

Output of this process is an array of samples of the residual samples y with elements yi, with i = 0..nS-1.

Depending on trType, the following applies:

–	If nS is equal to 4 and trType is equal to 1, the following applies:

yi = ∑j( dij * xj ) with i=0..3, j=0..3		(8-9)

- where the transform coefficient matrix d is specified as:

- -
		-
		-
		-

- -
		-
		-
		-

d =

{

{29  55  74  84},

{74  74   0 -74},

{84 -29 -74  55},

{55 -84  74 -29},

}