I would like to recommend the following two revisions to "JCTVC-G1103_d9.doc ":

- Revise the description of max_cu_qp_delta_depth syntax as shown below;

- And, replace "cu_qp_delta_enabled_flag " with " CUQPDeltaEnabledFlag"

max_cu_qp_delta_depth indicates the control granularity for QPY values for coding unit within a picture. The value of max_cu_qp_delta_depth shall be in the range of 0 to 4, inclusive.

If the max_cu_qp_delta_depth is 0, the variable CUQPDeltaEnabledFlag specifying the presence of cu_qp_delta in coding units of the current picture is set to 0 and the variable log2MinCUDQPSize specifying the minimum size of coding units that convey cu_qp_delta is set to Log2MaxCUSize+1. Otherwise (if the max_cu_qp_delta_depth is greater than 0), the variable CUQPDeltaEnabledFlag is set to 1 and the variable log2MinCUDQPSize is set to max(Log2MinCUSize, Log2MaxCUSize - (max_cu_qp_delta_depth - 1)).

-

The value of log2MinCUDQPSize should be larger than or equal to the CU size specified by the value of slice_granurality.