### Deblocking filter process

A conditional filtering process shall be performed on a treeblock basis after the completion of the picture construction process prior to deblocking filter process for the entire decoded picture (as specified in subclauses XXX and YYY), with all treeblocks in a picture processed in order of increasing treeblock addresses.

Vertical and horizontal edges in each  treeblock  are separately processed on a coding unit basis with the same order as decoding process. For each coding unit,  all vertical edges in a picture are filtered first, starting with the edge on the left-hand side of the coding unit proceeding through the edges towards the right-hand side of the coding unit in their geometrical order. Then,  all horizontal edges in a picture are filtered starting with the edge on the top of the coding unit proceeding through the edges towards the bottom of the coding unit in their geometrical order.

Sample values above of the current coding unit that may have already been modified by the filtering of horizontal edges of deblocking filter process operation on previous coding unit shall be used as inputs to the deblocking filter process on the current coding unit and may be further modified during the filtering of the current coding unit. Sample values to the left of the current coding unit shall be used as inputs to the deblocking filter process on the current coding unit and may be further modified during the filtering of the current coding unit. Sample values to the left of the current coding unit may be modified by the filtering of vertical edge and may be further modified by the filtering of horizontal edges.

Sample values modified during filtering of vertical edges are used as input for the filtering of the horizontal edges. For sample values modified by both filtering of horizontal edges and filtering of vertical edges, filtering of horizontal edges is applied after filtering of vertical edges.

The deblocking filter process shall be applied to all prediction unit edges and transform unit edges of a picture, except edges at the boundary of the picture, any edges for which the deblocking filter process is disabled by disable_deblocking_filter_flag, any edges coincide with tile boundaries when loop_filter_across_tiles_enabled_flag is equal to 0, and any edges coincide with slice boundaries of a particular slice when slice_loop_filter_across_slices_enabled_flag is equal to 0. For the transform units and prediction units with edges less than 8 samples in either vertical or horizontal direction, only the edges lying on the 8x8 sample grid are filtered.

When disable_deblocking_filter_flag is not equal to 1 and if qpprime_y_zero_transquant_bypass_flag is not equal to 1 or QP’Y is not equal to 0, the deblocking filter process is invoked as the following ordered steps for both vertical and horizontal edges in each coding unit with the same order as decoding process.

- The coding unit size nS is set equal to 1 << log2CbSize.

- The variables FilterInternalEdgesFlag, FilterLeftCuEdgeFlag and FilterTopCuEdgeFlag are derived as follows.

- The variable FilterInternalEdges is set equal to 1.

- If the left boundary of current coding unit is the left boundary of the picture, or if the left boundary of current coding unit is the left boundary of the tile and loop_filter_across_tiles_enabled_flag is equal to 0, or if the left boundary of current coding unit is the left boundary of the slice and slice_loop_filter_across_slices_enabled_flag is equal to 0, the variable FilterLeftCuEdgeFlag is set equal to 0, otherwise set equal to 1.

- If the top boundary of current coding unit is the top boundary of the picture, or if the top boundary of current coding unit is the top boundary of the tile and loop_filter_across_tiles_enabled_flag is equal to 0, or if the top boundary of current coding unit is the top boundary of the slice and slice_loop_filter_across_slices_enabled_flag is equal to 0, the variable FilterTopCuEdgeFlag is set equal to 0, otherwise set equal to 1.

- All elements of two-dimensional array of size (nS)x(nS), horEdgeFlags and verEdgeFlags are initialized to zero.

- The derivation process of transform unit boundary specified in subclause 8.7.2.1 are invoked with the luma location ( xB, yB ) set equal to ( 0, 0 ), the transform unit width log2TrafoWidth set equal to log2CbSize, the transform unit height log2TrafoHeight set equal to log2CbSize and the variable trafoDepth set equal to 0 as the inputs and the modified horEdgeFlags and verEdgeFlags as outputs.

- The derivation process of prediction unit boundary specified in subclause 8.7.2.2 are invoked with the coding unit size log2CbSize and the prediction partition mode PartMode as inputs, and the modified horEdgeFlags and verEdgeFlags as outputs.

- The derivation process of the boundary filtering strength specified in subclause  is invoked with the luma location ( xC, yC ), the coding unit size log2CbSize, horEdgeFlags and verEdgeFlags as inputs and an array of size (2)x(nS)x(nS), bS as output.

- The filtering process for coding unit specified in subclause 8.7.2.4 is invoked with the luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture, the coding unit size log2CbSize and the array bS as inputs and the modified reconstructued picture as output.

#### Derivation process of transform unit boundary

Inputs of this process are:

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current coding unit,

–	a variable log2TrafoWidth specifying the width of the current block,

–	a variable log2TrafoHeight specifying the height of the current block,

–	a variable trafoDepth.

Outputs of this process are:

–	two-dimensional arrays of (nS)x(nS), horEdgeFlags and verEdgeFlags.

Depending on split_transform_flag[ xB ][ yB ][ trafoDepth ], the following applies:

–	If split_transform_flag[ xB ][ yB ][ trafoDepth ] is equal to 1, the following ordered steps apply:

- The variables xB1, yB1, xB2, yB2, xB3 and yB3 are derived as follows.

  - If InterTUSplitDirection is equal to 2, the following applies.

  - The variable xB1 is set equal to xB + ( ( 1 << log2TrafoWidth ) >> 1 ).

  - The variable yB1 is set equal to yB.

  - The variable xB2 is set equal to xB.

  - The variable yB1 is set equal to yB + ( ( 1 << log2TrafoHeight ) >> 1 ).

  - The variable xB3 is set equal to xB1.

  - The variable yB3 is set equal to yB2.

  - The variable log2TrafoWidth1 is set equal to log2TrafoWidth − 1.

  - The variable log2TrafoHeight1 is set equal to log2TrafoHeight − 1.

  - Otherwise (InterTUSplitDirection is equal to 0 or 1), the following applies.

  - The variable xB1 is set equal to xB + ((1 << (log2TrafoWidth)) >> 2) * InterTUSplitDirection.

  - The variable yB1 is set equal to yB + ((1 << (log2TrafoHeight)) >> 2) * (1 − InterTUSplitDirection).

  - The variable xB2 is set equal to xB1 + ((1 << (log2TrafoWidth)) >> 2) * InterTUSplitDirection.

  - The variable yB2 is set equal to yB1 + ((1 << (log2TrafoHeight)) >> 2) * (1 − InterTUSplitDirection).

  - The variable xB3 is set equal to xB2 + ((1 << (log2TrafoWidth)) >> 2) * InterTUSplitDirection.

  - The variable yB3 is set equal to yB2 + ((1 << (log2TrafoHeight)) >> 2) * (1 − InterTUSplitDirection).

  - The variable log2TrafoWidth1 is set equal to (log2TrafoWidth − 2) * InterTUSplitDirection.

  - The variable log2TrafoHeight1 is set equal to (log2TrafoHeight − 2) * (1 − InterTUSplitDirection).

- The deriviation process of transform unit boundary as specified in this subclause is invoked with the luma location ( xB, yB ), the variable log2TrafoWidth set equal to log2TrafoWidth1, the variable log2TrafoHeight set equal to log2TrafoHeight1 and the variable trafoDepth1 set equal to trafoDepth + 1 as inputs and the outputs are the modified versions of two arrays, horEdgeFlags and verEdgeFlags.

- The deriviation process of transform unit boundary as specified in this subclause is invoked with the luma location ( xB1, yB1 ), the variable log2TrafoWidth set equal to log2TrafoSizeWidth1, the variable log2TrafoHeight set equal to log2TrafoHeight1 and the variable trafoDepth1 set equal to trafoDepth + 1 as inputs and the outputs are the modified versions of two arrays, horEdgeFlags and verEdgeFlags.

- The deriviation process of transform unit boundary as specified in this subclause is invoked with the luma location ( xB2, yB2 ), the variable log2TrafoWidth set equal to log2TrafoSizeWidth1, the variable log2TrafoHeight set equal to log2TrafoHeight1 and the variable trafoDepth1 set equal to trafoDepth + 1 as inputs and the outputs are the modified versions of two arrays, horEdgeFlags and verEdgeFlags.

- The deriviation process of transform unit boundary as specified in this subclause is invoked with the luma location ( xB3, yB3 ), the variable log2TrafoWidth1 set equal to log2TrafoSizeWidth1, the variable log2TrafoHeight set equal to log2TrafoHeight1 and the variable trafoDepth1 set equal to trafoDepth + 1 as inputs and the outputs are the modified versions of two arrays, horEdgeFlags and verEdgeFlags.

–	Otherwise (split_transform_flag[ xB ][ yB ][ trafoDepth ] is equal to 0), the following applies:

- If yB is equal to zero, horEdgeFlags[ xB + k ][ yB ] is set equal to FilterTopCuEdgeFlag, otherwise horEdgeFlags[ xB + k ][ yB ] is set equal to FilterInternalEdgesFlag for k = 0.. ( 1 << log2TrafoWidth ) − 1.

- If xB is equal to zero, verEdgeFlags[ xB ][ yB + k ] is set equal to FilterLeftCuEdgeFlag, otherwise verEdgeFlags[ xB ][ yB + k ] is set equal to FilterInternalEdgesFlag for k = 0.. ( 1 << log2TrafoHeight ) − 1.

#### Derivation process of prediction unit boundary

Inputs of this process are:

–	a variable log2CbSize specifying the coding unit size,

–	a prediction partition mode PartMode.

Outputs of this process are:

–	two-dimensional arrays of (nS)x(nS), horEdgeFlags and verEdgeFlags.

Depending on PartMode, the following applies:

–	If PartMode is equal to PART_2NxN or PART_NxN, horEdgeFlags[ k ][ 1 << ( log2CbSize − 1 ) ] is set equal to FilterInternalEdgesFlag for k = 0.. ( 1 << log2CbSize ) − 1.

–	If PartMode is equal to PART_Nx2N or PART_NxN, verEdgeFlags[ 1 << ( log2CbSize − 1 ) ][ k ] is set equal to FilterInternalEdgesFlag for k = 0.. ( 1 << log2CbSize ) − 1.

–	If PartMode is equal to PART_2NxnU, horEdgeFlags[ k ][ (1 << ( log2CbSize − 1 )) – (1 << ( log2CbSize − 2 )) ] is set equal to FilterInternalEdgesFlag for k = 0.. ( 1 << log2CbSize ) − 1.

–	If PartMode is equal to PART_2NxnD, horEdgeFlags[ k ][ (1 << ( log2CbSize − 1 )) + (1 << ( log2CbSize − 2 )) ] is set equal to FilterInternalEdgesFlag for k = 0.. ( 1 << log2CbSize ) − 1.

–	If PartMode is equal to PART_nLx2N, verEdgeFlags[ (1 << ( log2CbSize − 1 )) – (1 << ( log2CbSize − 2 )) ][ k ] is set equal to FilterInternalEdgesFlag for k = 0.. ( 1 << log2CbSize ) − 1.

–	If PartMode is equal to PART_nRx2N, verEdgeFlags[ (1 << ( log2CbSize − 1 )) + (1 << ( log2CbSize − 2 )) ][ k ] is set equal to FilterInternalEdgesFlag for k = 0.. ( 1 << log2CbSize ) − 1.

#### Derivation process of boundary filtering strength

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top-left luma sample of the current picture,

–	a variable log2CbSize specifying the size of the current coding unit,

–	a two-dimensional arrays of size (nS)x(nS), horEdgeFlags and verEdgeFlags.

Output of this process is an array of size (2)x(nS)x(nS), bS specifying the boundary filtering strength.

Let ( xEk, yEj ) with k = 0..nE−1 and j = 0..nE−1 specify a set of edge sample locations where nE is set equal to ( ( 1 << log2CbSize ) >> 2 ), xE0 = 0, yE0 = 0, xEk+1 = xEk + 4 and yEj+1 = yEj + 4.

For ( xEk, yEj ) with k = 0..nE−1 and j = 0..nE−1, the following applies.

- If horEdgeFlags[ xEk ][ yEj ] is equal to 1, for ( xEk+j, yEj ) with j=0, 1, the following applies.

- Set sample p0 = recPicture[ xC + xEk ][ yC + yEj − 1 ] and q0 = recPicture[ xC + xEk ][ yC + yEj ].

- The variable filterDir is set equal to 1.

- Otherwise, if verEdgeFlags[ xEk ][ yEj ] is equal to 1, for ( xEk, yEj +j) with j=0, 1, the following applies.

- Set sample p0 = recPicture[ xC + xEk − 1 ][ yC + yEj ] and q0 = recPicture[ xC + xEk ][ yC + yEj ].

- The variable filterDir is set equal to 0.

- Depending on the value of filterDir, the variable bS[ filterDir ][ xEk ][ yEj ] is derived as follows.

- If the following condition is true, the variable bS[ filterDir ][ xEk ][ yEj ] is set equal to 2.

- The sample p0 or q0 is in a coding unit coded with intra prediction mode

- Otherwise, if the block edge is also a transform unit edge and the following condition is true, the variable bS[ filterDir ][ xEk ][ yEj ] is set equal to 1.

- The sample p0 or q0 is in a transform unit which contains non-zero transform coefficient level.

- Otherwise, if any of the following conditions are true, the variable bS[ filterDir ][ xEk ][ yEj ] is set equal to 1.

- The prediction unit containing sample p0 has different reference pictures or a different number of motion vectors with the prediction unit containing the sample q0.

NOTE – The determination of whether the reference pictures used for the two prediction are the same or different is based on which pictures are referenced, without regard to whether a prediction is formed using an index into list 0 or an index into list 1, and also without regard to whether or not the index position within a reference picture list is different or not.

- If horEdgeFlags[ xEk ][ yEj ] is equal to 1 and yC + yEj − 1 is less than yCtb, the following applies.

    - One motion vector is used to predict the prediction unit containing sample recPicture[ xL ][ yC + yEj − 1 ], one motion vector is used to predict the prediction unit containing sample q0, and the absolute difference between the horizontal or vertical component of the motion vector used is greater than or equal to 4 in units of quarter luma samples, where xL is set equal to ( ( ( xC + xEk ) >> Log2MinCbSize ) << Log2MinCbSize ) + ( ( ( xC + xEk ) >> Log2MinCbSize ) & 1 ) * ( MinPuSize * 2 − 1) and MinPuSize is set equal to 1<<(Log2MinCbSize − 1).

  - Otherwise, the following applies.

    - One motion vector is used to predict the prediction unit containing sample p0, one motion vector is used to predict the prediction unit containing sample q0, and the absolute difference between the horizontal or vertical component of the motion vector used is greater than or equal to 4 in units of quarter luma samples.

- Two motion vectors are used to predict the prediction unit containing sample p0, two motion vectors are used to predict the prediction unit containing sample q0, and at least one of the motion vector pairs corresponding the same reference pictures and the different boundary samples p0 and q0 satisfies the following condition:

    - The absolute difference between the horizontal or vertical component of a motion vector used in the prediction of the two prediction units is greater than or equal to 4 in units of quarter luma samples.

- Otherwise, the variable bS[ filterDir ][ xEk ][ yEj ] is set equal to 0.

#### Filtering process

Inputs of this process are:

–	 a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a variable log2CbSize specifying the coding unit size,

–	an array bS specifying the boundary filtering strength.

Output of this process is:

–	modified reconstruction of the picture.

The filtering process consists of the following ordered steps:

- For all coding unit, the filtering process for vertical edge specified in 8.7.1.4.1 is invoked with verticalEdgeFlag equal to 1.

- For all coding unit, the filtering process for horizontal edge specified in 8.7.1.4.2 is invoked with verticalEdgeFlag equal to 0.

##### Filtering process for vertical edge in coding unit

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a variable log2CbSize specifying the coding unit size,

–	an array bS specifying the boundary filtering strength.

Output of this process is:

–	modified reconstruction of the picture.

The filtering process for luma edges in the current coding unit consists of the following ordered steps:

- The variable nD is set equal to 1 << ( log2CbSize − 3 ).

- For xDk set equal to xC+( k << 3 ), k=0..nD − 1, the following applies:

- For yDm set equal to yC+( m << 2 ), m=0..nD*2 − 1, the following ordered steps apply:

- Boundary filtering strength bSVer is derived as follows:

bSVer = bS[ 0 ][ xDk ][ yDm ]		(8-1)

- When bSVer is greater than 0, decision process for luma block edge in subclause 8.7.2.4.9 is invoked with the luma location of the coding unit ( xC, yC ), the luma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 1, and the boundary filtering strength bSVer as inputs, the decision dE, dEp, dEq as output.

- When bSVer is greater than 0, filtering process for luma block edge in subclause 8.7.2.4.10 is invoked with the luma location of the coding unit ( xC, yC ), the luma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 1, the boundary filtering strength bSVer, the decision dE, dEp, dEq as inputs and the modified luma picture as outputs.

-

The filtering process for chroma edges in the current coding unit consists of the following ordered steps:

- The variable nD is set equal to 1 << ( Max( log2CbSize, 4 ) − 4 ).

- For xDk set equal to ( xC / 2 )+( k << 3 ), k=0..nD − 1, the following applies:

- For yDm set equal to ( yC / 2)+( m << 2 ), m=0..nD*2 − 1, the following ordered steps apply:

- Boundary filtering strength bSVer is derived as follows:

bSVer = bS[ 0 ][ xDk*2 ][ yDm*2 ] 		(8-3)

- When bSVer is greater than 1, filtering process for chroma block edge in subclause 8.7.2.4.11 is invoked with the chroma location of the coding unit ( xC/2, yC/2 ), the chroma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 1, a chroma component index cIdx set equal to 1 and the boundary filtering strength bSVer as inputs and the modified chroma picture buffer as output.

- When bSVer is greater than 1, filtering process for chroma block edge in subclause 8.7.2.4.11 is invoked with the chroma location of the coding unit ( xC/2, yC/2 ), the chroma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 1, a chroma component index cIdx set equal to 2 and the boundary filtering strength bSVer as inputs and the modified chroma picture as output.

##### -

##### Filtering process horizontal edge for coding unit

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a variable log2CbSize specifying the coding unit size,

–	an array bS specifying the boundary filtering strength.

Output of this process is:

–	modified reconstruction of the picture.

The filtering process for luma edges in the current coding unit consists of the following ordered steps:

- The variable nD is set equal to 1 << ( log2CbSize − 3 ).

- For yDm set equal to yC+( m << 3 ), m=0..nD − 1, the following applies:

- For xDk set equal to xC + ( k << 2 ), k = 0..nD*2 - 1, the following ordered steps apply:

- Boundary filtering strength bSHor is derived as follows:

bSHor = bS[ 1 ][ xDk ][ yDm ]	 	(8-5)

- When bSHor is greater than 0, decision process for luma block edge in subclause 8.7.2.4.93 is invoked with the luma location of the coding unit ( xC, yC ), the luma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 0, and the boundary filtering strength bSHor as inputs and the decision dE, dEp, dEq as output.

- For yDm set equal to yC+( m << 3 ), m=0..nD - 1, the following applies:

- For xDk set equal to xC+( k << 2 ), k=0..nD*2 − 1, the following ordered steps apply:

    - When bSHor is greater than 0, filtering process for luma block edge in subclause 8.7.2.4.10 is invoked with the luma location of the coding unit ( xC, yC ), the luma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 0, the boundary filtering strength bsHor, the decisions dEp, dEp, dEq as inputs and the modified luma picture as output.

The filtering process for chroma edges in the current coding unit consists of the following ordered steps:

- The variable nD is set equal to 1 << ( Max( log2CbSize, 4 ) − 4 ).

- For yDm set equal to ( yC / 2 )+( m << 3 ), m=0..nD − 1, the following applies:

- For xDk set equal to ( xC / 2 )+( k << 2 ), k=0..nD*2 − 1, the following ordered steps apply:

- Boundary filtering strength bSHor is derived as follows:

bSHor = bS[ 1 ][ xDk*2 ][ yDm*2 ] 		(8-6)

- When bSHor is greater than 1, filtering process for chroma block edge in subclause 8.7.2.4.11 is invoked with the chroma location of the coding unit ( xC/2, yC/2 ), the chroma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 0, a chroma component index cIdx set equal to 1 and the boundary filtering strength bSHor as inputs and the modified chroma picture as output.

- When bSHor is greater than 1, filtering process for chroma block edge in subclause 8.7.2.4.11 is invoked with the chroma location of the coding unit ( xC/2, yC/2 ), the chroma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 0, a chroma component index cIdx set equal to 2 and the boundary filtering strength bSHor as inputs and the modified chroma picture as output.

##### Decision process for luma block edge

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top left luma sample of the current coding unit,

–	a variable verticalEdgeFlag,

–	a variable bS specifying the boundary filtering strength,

Output of this process is:

–	variables dE, dEp, dEq containing decisions,

This process is invoked only when bS is not equal to 0.

Let s’ represent the luma sample array recPictureL of the current picture.

Depending on pcm_flag, a variable β is specified as follows:

A variable qPL is derived as follows:

- If pcm_flag of the coding unit containing the sample p00 is 1, QPp is set equal to 0

- Otherwise QPP   is set equal to  luma quantization parameter of the coding unit

- If pcm_flag of the coding unit containing the sample q00 is 1, QPY is set equal to 0

- Otherwise QPY  is set equal to  luma quantization parameter of the coding unit

- qPL = ( ( QPY + QPP + 1 ) >> 1 )	where QPP is specified by the QPY value of the coding unit containing the sample p0,0.	(8-7)

-

A variables β is specified as Table 8-1 with luma quantization parameter Clip3( 0, 51, qPL + ( beta_offset_div2 << 1 ) ) as input.

A variable tC is specified as Table 8-1 with luma quantization parameter Clip3( 0, 55, qPL + 2*( bS − 1) + ( tc_offset_div2 << 1) ) as input.

Depending on verticalEdgeFlag, the following applies:

–	If verticalEdgeFlag is equal to 1, the following ordered steps apply:

- The sample values pi,k and qi,k with i = 0..3 and k = 0, 3 are derived as follows:

qi,k = s’[ xC + xB +i, yC + yB + k ]		(8-9)

pi,k = s’[ xC + xB – i − 1, yC + yB + k ]		(8-10)

- The variables dpq0, dpq3, dp, dq are derived as follows:

dp0 = | p2,0 − 2*p1,0 + p0,0 |

dp3 =| p2,3 − 2*p1,3 + p0,3 | 		(8-11)

dq0 = | q2,0 − 2*q1,0 + q0,0 | 		(8-12)

dq3 = | q2,3 − 2*q1,3 + q0,3 |		(8-13)

dpq0 = dp0 + dq0		(8-14)

dpq3 = dp3 + dq3		(8-15)

dp = dp0 + dp3		(8-16)

dq = dq0 + dq3		(8-17)

- The variable d is derived as follows:

d = dpq0 + dpq3		(8-18)

- The variables dE, dEp and dEq are set equal to 0.

- if d  is less than β, the following ordered steps apply:

- For the sample location ( xC + xB, yC + yB ), the decision process for a luma sample specified in subclause 8.7.2.4.12 is invoked with sample values pi,0, qi,0, the variables 2*dpq0, β and tC as inputs and a decision dSam0 as output.

- For the sample location ( xC + xB, yC + yB + 3), the decision process for a luma sample specified in subclause 8.7.2.4.12 is invoked with sample values pi,3, qi,3, the variables 2*dpq3, β and tC as inputs and a decision dSam3 as output.

- The variable dE is set equal to 1.

- If dSam0 is equal to 1 and dSam3 is equal to 1, the variable dE is set equal to 2.

- If dp is less than ( β + ( β >> 1 ) ) >> 3, the variable dEp is set equal to 1.

- If dq is less than ( β + ( β >> 1 ) ) >> 3, the variable dEq is set equal to 1.

–	Otherwise (verticalEdgeFlag is equal to 0), the following ordered steps apply:

- The sample values pi,k and qi,k with i = 0..3 and k = 0, 3 are derived as follows:

qi,k = s’[ xC + xB +k, yC + yB + i ]		(8-19)

pi,k = s’[ xC + xB +k, yC + yB – i − 1 ]		(8-20)

- The variables dpq0, dpq3, dp, dq are derived as follows:

dp0 = | p2,0 − 2*p1,0 + p0,0 | 		(8-21)

dp3 =| p2,3 − 2*p1,3 + p0,3 | 		(8-22)

dq0 = | q2,0 − 2*q1,0 + q0,0 | 		(8-23)

dq3 = | q2,3 − 2*q1,3 + q0,3 |		(8-24)

dpq0 = dp0 + dq0		(8-25)

dpq3 = dp3 + dq3		(8-26)

dp = dp0 + dp3		(8-27)

dq = dq0 + dq3		(8-28)

- The variables d is derived as follows:

d = dpq0 + dpq3		(8-29)

- The variables dE, dEp and dEq are set equal to 0.

- if d is less than β, the following ordered steps apply:

- For the sample location ( xC + xB, yC + yB ), the decision process for a luma sample specified in subclause 8.7.2.4.12 is invoked with sample values pi,0, qi,0 the variables 2*dpq0, β and tC as inputs and a decision dSam0 as output.

- For the sample location ( xC + Xb + 3, yC + yB ), the decision process for a luma sample specified in subclause 8.7.2.4.12 is invoked with sample values pi,3, qi,3, the variables 2*dpq3, β and tC as inputs and a decision dSam3 as output.

- The variable dE is set equal to 1.

- If dSam0 is equal to 1 and dSam3 is equal to 1, the variable dE is set equal to 2.

- If dp is less than ( β + ( β >> 1 ) ) >> 3, the variable dEp is set equal to 1.

- If dq is less than ( β + ( β >> 1 ) ) >> 3, the variable dEq is set equal to 1.

##### Filtering process for luma block edge

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top left luma sample of the current coding unit,

–	a variable verticalEdgeFlag,

–	a variable bS specifying the boundary filtering strength,

–	variables dE, dEp, dEq containing decisions,

–	a variable β,

–	a variable tC,

Output of this process is:

–	modified reconstruction of the picture.

This process is invoked only when bS is not equal to 0.

Let s’ represent the luma sample array recPictureL of the current picture.

Depending on verticalEdgeFlag, the following applies:

–	If verticalEdgeFlag is equal to 1, the following ordered steps apply:

- The sample values pi,k and qi,k with i = 0..3 and k = 0..3 are derived as follows:

qi,k = s’[ xC + xB +i, yC + yB + k ]		(8-30)

pi,k = s’[ xC + xB – i − 1, yC + yB + k ]		(8-31)

- If dE is not equal to 0, for each sample location ( xC + xB, yC + yB + k ), k = 0..3, the following ordered steps apply:

- The filtering process for a luma sample specified in subclause 8.7.2.4.13 is invoked with sample values pi,k, qi,k with i = 0..3, the decision dE, variables dEp and dEq, the variable tC as inputs and the number of filtered samples nDp and nDq from each side of the block boundary, and the filtered sample values pi’ and qj’ as outputs.

- The filtered sample values pi’ and qj’ with i = 0..nDp − 1, j = 0..nDq − 1 replace the corresponding samples inside the sample array s’ as follows:

s’[ xC + xB +j, yC + yB + k ] = qj’		(8-32)

s’[ xC + xB – i − 1, yC + yB + k ] = pi’		(8-33)

–	Otherwise (verticalEdgeFlag is equal to 0), the following ordered steps apply:

- The sample values pi,k and qi,k with i = 0..3 and k = 0..7 are derived as follows:

qi,k = s’[ xC + xB +k, yC + yB + i ]		(8-34)

pi,k = s’[ xC + xB +k, yC + yB – i − 1 ]		(8-35)

-

-

- If dE is not equal to 0, for each sample location ( xC + xB + k, yC + yB ), k = 0.. 7, the following ordered steps apply:

- The filtering process for a luma sample specified in subclause 8.7.2.4.13 is invoked with sample values pi,k, qi,k with i = 0..3, decision dE, variables dEp and dEq, the variable tC as inputs and the number of filtered samples nDp and nDq from each side of the block boundary and the filtered sample values pi’ and qj’ as outputs.

- The filtered sample values pi’ and qj’ with i = 0..nDp − 1, j = 0..nDq − 1 replace the corresponding samples inside the sample array s’ as follows:

s’[ xC + xB + k, yC + yB + j ] = qj’		(8-38)

s’[ xC + xB + k, yC + yB – i − 1 ] = pi’		(8-39)

##### Filtering process for chroma block edge

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left chroma sample of the current coding unit relative to the top left chroma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left chroma sample of the current block relative to the top left chroma sample of the current coding unit,

–	a variable verticalEdgeFlag,

–	a variable bS specifying the boundary filtering strength,

–	a variable cIdx specifying the chroma component index.

Output of this process is:

–	modified reconstruction of the picture.

This process is invoked only when bS is greater than 1.

Let s’ be a variable specifying chroma sample array which is derived as follows.

–	If cIdx is equal to 1, s’ represents the chroma sample array recPictureCb of the current picture.

–	Otherwise (cIdx is equal to 2), s’ represents the chroma sample array recPictureCr of the current picture.

A variable qPL is derived as follows:

- If pcm_flag of the coding unit containing the sample p00 is 1, QPp is set equal to 0

- Otherwise QPP   is set equal to  luma quantization parameter of the coding unit

- If pcm_flag of the coding unit containing the sample q00 is 1, QPY is set equal to 0

- Otherwise QPY is set equal to  luma quantization parameter of the coding unit

qPL = ( ( QPY + QPP + 1 ) >> 1 )

where QPP is specified by the QPY value of the coding unit containing the sample p0,0.	(8-40)

A variable tC is specified as Table 8-1 with luma quantization parameter Clip3( 0, 53, qPL + 2*(bS − 1) + (tc_offset_div2 << 1 ) ) as input.

Depending on verticalEdgeFlag, the following applies:

–	If verticalEdgeFlag is equal to 1, for each sample location ( xC + xB, yC + yB + k ), k = 0..3, the following ordered steps apply:

- The sample values pi and qi with i = 0..1 are derived as follows:

qi = s’[ xC + xB +i, yC + yB + k ]		(8-41)

pi = s’[ xC + xB – i − 1, yC + yB + k ]		(8-42)

- The following ordered steps apply:

- The filtering process for a sample specified in subclause 8.7.2.4.14 is invoked with sample values pi, qi, with i = 0..1, the boundary filtering strength bS and the variable tC as inputs and the filtered sample values p0’ and q0’ as outputs.

- The filtered sample values p0’ and q0’ replace the corresponding samples inside the sample array s’ as follows:

s’[ xC + xB , yC + yB + k ] = q0’		(8-43)

s’[ xC + xB − 1, yC + yB + k ] = p0’		(8-44)

–	Otherwise (verticalEdgeFlag is equal to 0), the following ordered steps apply:

- The sample values pi and qi with i = 0..1 and k = 0..3 are derived as follows:

qi = s’[ xC + xB +k, yC + yB + i ]		(8-45)

pi = s’[ xC + xB +k, yC + yB – i − 1 ]		(8-46)

-

-

- If bS is greater than 1, for each sample location ( xC + xB + k, yC + yB ), k = 0..3, the following ordered steps apply:

- The filtering process for a sample specified in subclause 8.7.2.4.14 is invoked with sample values pi, qi, with i = 0..1, the boundary filtering strength bS and the variable tC as inputs and the filtered sample values p0’ and q0’ as outputs.

- The filtered sample values p0’ and q0’ replace the corresponding samples inside the sample array s’ as follows:

s’[ xC + xB +k, yC + yB ] = q0’		(8-49)

s’[ xC + xB +k, yC + yB − 1 ] = p0’		(8-50)

##### Decision process for a luma sample

Inputs of this process are:

–	sample values, pi and qi with i = 0..3,

–	variables d, β and tC.

Output of this process is:

–	a variable dSam containing a decision

The variable dSam is specified as follows:

–	If d is less than ( β >> 2 ), | p3 – p0 | + | q0 – q3 | is less than ( β >> 3 ) and | p0 – q0 | is less than ( 5*tC + 1 ) >> 1, dSam is set equal to 1.

–	Otherwise, dSam is set equal to 0.

##### Filtering process for a luma sample

Inputs of this process are:

–	sample values, pi and qi with i = 0..3,

–	variables dE, dEp and dEq containing decisions,

–	a variable tC.

Output of this process is:

–	number of filtered samples nDp and nDq,

–	filtered sample values, pi’ and qj’ with i = 0..nDp − 1, j = 0..nDq − 1

Depending on dE, the following applies:

–	When the variable dE is equal to 2, the following strong filtering applies while nDp and nDq are set equal to 3:

p0’ = Clip3( p0−2*tc, p0+2*tc, ( p2 + 2*p1 + 2*p0 + 2*q0 + q1 + 4 ) >> 3 )	(8-51)

p1’ = Clip3( p1−2*tc, p1+2*tc, ( p2 + p1 + p0 + q0 + 2 ) >> 2 )	(8-52)

p2’ = Clip3( p2−2*tc, p2+2*tc, ( 2*p3 + 3*p2 + p1 + p0 + q0 + 4 ) >> 3 )	(8-53)

q0’ = Clip3( q0−2*tc, q0+2*tc, ( p1 + 2*p0 + 2*q0 + 2*q1 + q2 + 4 ) >> 3 )	(8-54)

q1’ = Clip3( q1−2*tc, q1+2*tc, ( p0 + q0 + q1 + q2 + 2 ) >> 2 )	(8-55)

q2’ = Clip3( q2−2*tc, q2+2*tc, ( p0 + q0 + q1 + 3*q2 + 2*q3 + 4 ) >> 3 )	(8-56)

–	Otherwise, the following weak filtering applies while nDp and nDq are set equal to 0:

 = ( 9 * ( q0 –  p0 ) − 3 * ( q1 – p1 ) + 8 ) >> 4		(8-57)

  - When abs() is less than tc*10, the following ordered steps apply:

    - The filtered sample values p0’ and q0’ are specified as follows:

 = Clip3( -tc, tc,  )		(8-58)

p0’ = Clip1Y( p0 +  )		(8-59)

q0’ = Clip1Y( q0 -  )		(8-60)

    - If dEp is equal to 1, the filtered sample value pi’ is specified as follows:

p = Clip3( -(tc >> 1), tc >> 1, ( ( ( p2 + p0 + 1 ) >> 1 ) – p1 +  ) >>1 )	(8-61)

pi’ = Clip1Y( p1 + p )		(8-62)

    - If dEq is equal to 1, the filtered sample value qi’ is specified as follows:

q = Clip3( -(tc >> 1), tc >> 1, ( ( ( q2 + q0 + 1 ) >> 1 ) – q1 –  ) >>1 )	(8-63)

qi’ = Clip1Y( q1 + q )		(8-64)

    - nDp is set equal to dEp1+1 and nDq is set equal to dEq1+1.

Each of the filtered sample values, pi’ with i = 0..nDp−1, is substituted by the corresponding input sample value pi if all of the following conditions are true.

–	pi is a sample of an I_PCM block.

–	pcm_loop_filter_disable_flag value is equal to 1.

Similarly, each of the filtered sample values, qj’ with j = 0..nDq−1, is substituted by the corresponding input sample value qj if all of the following conditions are true.

–	qj is a sample of an I_PCM block.

–	pcm_loop_filter_disable_flag value is equal to 1.

Table 8-1 – Derivation of threshold variables β and tC from input Q

| Q | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| β | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 6 | 7 | 8 |
| tC | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| Q | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 |
| β | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 20 | 22 | 24 | 26 | 28 | 30 | 32 | 34 | 36 |
| tC | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 | 4 | 4 | 4 |
| Q | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 |  |
| β | 38 | 40 | 42 | 44 | 46 | 48 | 50 | 52 | 54 | 56 | 58 | 60 | 62 | 64 | 64 | 64 | 64 | 64 |  |
| tC | 5 | 5 | 6 | 6 | 7 | 8 | 9 | 9 | 10 | 10 | 11 | 11 | 12 | 12 | 13 | 13 | 14 | 14 |  |

##### Filtering process for a chroma sample

Inputs of this process are:

–	sample values, pi and qi with i = 0..1,

–	a variable tC.

Output of this process is:

–	The filtered sample values, p0’ and q0’.

The filtered sample values p0’ and q0’ are derived by

 = Clip3( -tC, tC, ( ( ( ( q0 – p0 ) << 2 ) + p1 – q1 + 4 ) >> 3 ) )	(8-65)

p0’ = Clip1C( p0 +  )		(8-66)

q0’ = Clip1C( q0 -  )		(8-67)

The filtered sample value, p0’ is substituted by the corresponding input sample value p0 if all of the following conditions are true.

–	p0 is a sample of an I_PCM block.

–	pcm_loop_filter_disable_flag value is equal to 1.

Similarly, the filtered sample value, q0’ is substituted by the corresponding input sample value q0 if all of the following conditions are true.

–	q0 is a sample of an I_PCM block.

–	pcm_loop_filter_disable_flag value is equal to 1.