# Bug fix to the current text

1  Bug fix to 8.7.1.4.1 Decision process for luma block edge

Replace the following text

" Let s’ represent the luma sample array recPictureL of the current picture.

A variable qPL is derived as follows:

qPL = ( ( QPY + QPP + 1 ) >> 1 )

where QPP is specified by the QPY value of the coding unit containing the sample p0,0.	(8-1)"

by

"Let s’ represent the luma sample array s’ of the current picture.

The sample value q0,0 is derived as follows:

q0,0 = s’ [ xC + xB, yC + yB ]

If verticalEdgeFlag is equal to 1, the sample value p0,0 is derived as follows:

p0,0 = s’ [ xC + xB – 1, yC + yB ]

Otherwise (verticalEdgeFlag is equal to 0),

p0,0 = s’ [ xC + xB, yC + yB – 1]

The variables QPQ and QPP are set equal to the QPY values of the coding unit containing the sample q0,0 and p0,0, respectively.

If pcm_enabled_flag is equal to 1, the variables QPQ and QPP are modified as follows:

–	If pcm_flag of the coding unit containing the sample q0,0 is equal to 1, QPQ is set equal to 0;

- If pcm_flag of the coding unit containing the sample p0,0 is equal to 1, QPP is set equal to 0.

A variable qPL is derived as follows:

qPL = ( ( QPQ + QPP + 1 ) >> 1 ) 		 					(8-2)

A variable qPL is derived as follows:

qPL = ( ( QPQ + QPP + 1 ) >> 1 )

where QPP is specified by the QPY value of the coding unit containing the sample p0,0.	(8-3)"

2	Bug fix to 8.7.1.4.2 Filtering process for luma block edge

Replace the following text

" Depending on pcm_flag, a variable β is specified as follows:

–	If pcm_flag is equal to 1, the variables β is specified as Error: Reference source not found with luma quantization parameter 0 as input.

–	Otherwise, the variables β is specified as Error: Reference source not found with luma quantization parameter Clip3( 0, 51, qPL +  ( beta_offset_div2 << 1 ) ) as input."

by

" The variables β is specified as Error: Reference source not found with luma quantization parameter Clip3( 0, 51, qPL +  ( beta_offset_div2 << 1 ) ) as input.

Depending on pcm_flag, a variable β is specified as follows:

–	If pcm_flag is equal to 1, the variables β is specified as Error: Reference source not found with luma quantization parameter 0 as input.

–	Otherwise, the variables β is specified as Error: Reference source not found with luma quantization parameter Clip3( 0, 51, qPL +  ( beta_offset_div2 << 1 ) ) as input."

3	Bug fix to 8.7.1.4.3 Filtering process for chroma block edge

Replace the following text

" Let s’ be a variable specifying chroma sample array which is derived as follows.

–	If cIdx is equal to 1, s’ represents the chroma sample array recPictureCb of the current picture.

–	Otherwise (cIdx is equal to 2), s’ represents the chroma sample array recPictureCr of the current picture.

A variable qPL is derived as follows:

qPL = ( ( QPY + QPP + 1 ) >> 1 )

where QPP is specified by the QPY value of the coding unit containing the sample p0,0.	(8-4)"

by

" Let s’ be a variable specifying chroma sample array which is derived as follows.

–	If cIdx is equal to 1, s’ represents the chroma sample array recPictureCb of the current picture.

–	Otherwise (cIdx is equal to 2), s’ represents the chroma sample array recPictureCr of the current picture.

The sample value q0,0 is derived as follows:

q0,0 = s’ [ xC + xB, yC + yB ]

If verticalEdgeFlag is equal to 1, the sample value p0,0 is derived as follows:

p0,0 = s’ [ xC + xB – 1, yC + yB ]

Otherwise (verticalEdgeFlag is equal to 0),

p0,0 = s’ [ xC + xB, yC + yB – 1]

The variables QPQ and QPP are set equal to the QPY values of the coding unit containing the sample q0,0 and p0,0, respectively.

If pcm_enabled_flag is equal to 1, the variables QPQ and QPP are modified as follows:

–	If pcm_flag of the coding unit containing the sample q0,0 is equal to 1, QPQ is set equal to 0;

- If pcm_flag of the coding unit containing the sample p0,0 is equal to 1, QPP is set equal to 0.

A variable qPL is derived as follows:

qPL = QPUV( ( QPQ + QPP + 1 ) >> 1 ) 		(8-5)

A variable qPL is derived as follows:

qPL = ( ( QPY + QPP + 1 ) >> 1 )

where QPP is specified by the QPY value of the coding unit containing the sample p0,0.	(8-6)

where QPUV as specified in Table 8-XX based on the index qPL.

- Table 8-xx – Specification of QPUV as a function of qPL

| qPL | <30 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPUV | = qPL | 29 | 30 | 31 | 32 | 32 | 33 | 34 | 34 | 35 | 35 | 36 | 36 | 37 | 37 | 37 | 38 | 38 | 38 | 39 | 39 | 39 | 39 |

"