| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0412_r1 m23288 |
| --- | --- |

| Title: | AHG21: Reference picture list restrictions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rickard Sjöberg Jonatan Samuelsson Per Wennersten Ericsson Research Färögatan 6 16480 Stockholm | Tel: Email: | +46 761153563 rickard.sjoberg@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This document proposes to add a flag identical_ref_pic_lists_flag to the SPS to enable indicating whether the reference picture lists for all slices that belong to the same picture and have the same slice type are identical or not. If this flag is set, the decoder may perform reference picture list construction once per picture regardless of the number of slices that are used. This document claims that the list construction process is a burden for the decoder if there are many slices for each picture since list construction is generally not part of decoding hardware but done on relatively slow general purpose processors.

This document also proposes to add a flag to the SPS to signal whether there is any reference picture list modification syntax in the bitstream at all. Besides indicating to the decoder that there are no modifications of the lists, the presence of reference picture list modification syntax is conditioned on this flag, saving a few bits.

This document finally proposes that identical_ref_pic_lists_flag shall be equal to 1 for HEVC profiles targeting broadcast TV.

# Introduction

The picture list construction mechanisms in HEVC are very flexible. Full flexibility is provided by using ref_pic_list_modification() and ref_pic_list_combination() syntax. Moreover, list construction is done separately for each slice, enabling the use of different lists within the same picture.

Although the possibility of using different lists for different parts of the picture is nice, it is very rare to see this used in real-life bitstreams. Also, the list construction process is a burden for the decoder if there are many slices for each picture, since list construction is generally not part of decoding hardware but done on relatively slow general purpose processors.

# Proposal

We propose to add a flag to the SPS to indicate whether different lists are used for the same slice type in any picture in the bitstream or not. If this flag is set to 1, the lists for all slices of the same type in the same picture are the same. Different pictures may have different lists but list construction for all slices of the same type in one picture is identical. This means that a decoder can perform the list construction once for each picture no matter how many slices there are. Additionally, conditioned on this first flag, we propose to add another flag to indicate whether there are any list modifications at all. If there are not, we do not need to include modification syntax in the slice header.

## Syntax and semantics

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| num_reorder_frames | u(1) |
| identical_ref_pic_lists_flag | u(1) |
| if(identical_ref_pic_lists_flag = = 1) { |  |
| ref_pic_list_modification_not_present_flag | u(1) |
| } |  |
| ... |  |
| } |  |

identical_ref_pic_lists_flag equal to 1 indicates that all slices of the same slice type that belong to the same picture have identical reference picture lists. identical_ref_pic_lists_flag equal to 0 indicates that there may be slices of the same slice type that belong to the same picture that have different reference picture lists. num_ref_idx_active_override_flag, num_ref_idx_l0 active_minus1, num_ref_idx_l1_active_minus1, ref_pic_list_combination_flag and num_ref_idx_lc_active_minus1 shall when present be identical for all slices of the same slice type that belong to the same picture when identical_ref_pic_lists_flag is equal to 1.

ref_pic_list_modification_not_present_flag equal to 1 specifies that the syntax elements ref_pic_list_modification_flag_l0, ref_pic_list_modification_flag_l1 and ref_pic_list_modification_flag_lc are not present. no_ref_pic_list_modification_present_flag equal to 0 specifies that syntax elements ref_pic_list_modification_flag_l0, ref_pic_list_modification_flag_l1 and ref_pic_list_modification_flag_lc are present. When ref_pic_list_modification_present_flag is not present, it shall be inferred to be equal to 0.

| slice_header ( ) { | Descriptor |
| --- | --- |
| ... |  |
| if( ref_pic_list_modification_not_present_flag = = 0 ) |  |
| ref_pic_list_modification() |  |
| ref_pic_list_combination() |  |
| ... |  |
| } |  |

ref_pic_list_modification_flag_l0 equal to 1 specifies that the syntax element list_modification_idc is present for specifying reference picture list 0. ref_pic_list_modification_flag_l0 equal to 0 specifies that this syntax element is not present.

When ref_pic_list_modification_flag_l0 is equal to 1, the number of times that list_modification_idc is not equal to 2 following ref_pic_list_modification_flag_l0 shall not exceed num_ref_idx_l0_active_minus1 + 1.

When ref_pic_list_modification_flag_l0 is not present, it shall be inferred to be equal to 0.

ref_pic_list_modification_flag_l1 equal to 1 specifies that the syntax element list_modification_idc is present for specifying reference picture list 1. ref_pic_list_modification_flag_l1 equal to 0 specifies that this syntax element is not present.

When ref_pic_list_modification_flag_l1 is equal to 1, the number of times that list_modification_idc is not equal to 2 following ref_pic_list_modification_flag_l1 shall not exceed num_ref_idx_l1_active_minus1 + 1.

When ref_pic_list_modification_flag_l1 is not present, it shall be inferred to be equal to 0.

equal to 1 indicates that th_flag is not present, it shall be inferred to be equal to 0.

| ref_pic_list_combination( ) { | Descriptor |
| --- | --- |
| if( slice_type = = B ) { |  |
| ref_pic_list_combination_flag | u(1) |
| if( ref_pic_list_combination_flag ) { |  |
| num_ref_idx lc_active_minus1 | ue(v) |
| if( ref_pic_list_modification_not_present_flag = = 0 ) { |  |
| ref_pic_list_modification_flag_lc | u(1) |
| if( ref_pic_list_modification_flag_lc ) { |  |
| for ( i =0; i <= num_ref_idx_lc_active_minus1; i++ ) { |  |
| pic_from_list_0_flag | u(1) |
| ref_idx_list_curr | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

## Profiling

We propose that the HEVC profiles targeting broadcast TV restricts the use of identical_ref_pic_lists_flag as follows:

- The value of identical_ref_pic_lists_flag shall be equal to 1.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).